Python-Based Design and Evaluation of an Approximate MAC (AxMAC)

for Multi-Precision DNN Inference

Jiabo Zhang; Yuxuan Wang

1. Overview

This project redesigns a multi-precision approximate MAC (AxMAC) study around the research arc baseline → deficiency → improvement. The baseline reproduces the truncated-multiplier (Mahdiani et al., IEEE TCAS-I 2010) and ACA adder (Kahng & Kang, DAC 2012). Two deficiencies are identified and fixed with quantified gains. A modernization pass extends the study to FP8 formats (E4M3/E5M2, Micikevicius et al. 2022), the precision used in current LLM inference hardware (NVIDIA H100/H200, AMD MI300X).

2. Deficiencies Identified

D1 — Truncation is biased. Zeroing the low K product bits creates a one-signed error in [0, 2ᵏ); its mean is ≈2ᵏ⁻¹. Over N MACs this bias accumulates coherently (∝N), while a zero-mean error grows only ∝√N. At N = 4096 the truncation error reaches 26 617 vs. 14 for stochastic rounding — a 1 900× gap confirmed experimentally.

D2 — Uniform K wastes inter-layer sensitivity. A single global K ignores that layers differ in error tolerance and MAC count (the demo MLP’s first layer carries 95.8 % of all MACs).

3. Contributions

A — Error-compensated multiplier (rounding knob in approx_mac.py): trunc (baseline), round (add 2ᵏ⁻¹ before masking; Schulte & Swartzlander 1993), stochastic (Gupta et al. 2015). Result: INT8 K = 6 per-MAC bias drops from +29.9 (trunc) to −1.7 (round) to −0.8 (stochastic). Deterministic round nearly matches stochastic at zero hardware cost (one constant addend vs. a per-MAC RNG/LFSR).

B — Sensitivity-driven non-uniform K allocation (sensitivity.py): layer sensitivity is measured as logit-NRMSE under a probe K; the budget is distributed inversely proportional to sensitivity, weighted by MAC count. Result: at matched inference energy, sensitivity-driven allocation cuts logit NRMSE by up to 48 % vs. uniform K on a 256→256→256→256→10 INT8 MLP.

4. Modernization (FP8 Era)

FP8 formats E4M3 and E5M2 are added to exact_mac.py and swept in the Pareto analysis. E4M3 K = 0 costs 0.50 pJ/MAC vs. 1.18 pJ for FP16 — a 2.4× energy reduction before any truncation. The cross-format Pareto front (NRMSE vs. float64 reference) spans INT4 K = 1 (0.098 pJ, NRMSE ≈ 0.82) through FP32 K = 11 (3.29 pJ, NRMSE = 1.4×10⁻⁷), with per-format fronts for all eight precisions.

5. Technical Approach

| Module | Role |
| --- | --- |
| exact_mac.py | Bit-accurate reference: INT4/8/16, FP8 E4M3/E5M2, FP16, BF16, FP32 |
| approx_mac.py | Truncated multiplier + ACA adder; rounding ∈ {trunc, round, stochastic} |
| power_model.py | Switching-activity model; stochastic adds per-MAC RNG cost (LFSR) |
| accuracy_eval.py | RMSE, MED, NMED, bias over random pairs per (precision, K, rounding) |
| sensitivity.py | Layer sensitivity probe + non-uniform K allocation (Contribution B) |
| pareto.py | Cross-format Pareto on (energy pJ, NRMSE vs float64); per-format fronts |
| main.py | Full sweep driver; produces 10+ CSV result files |
| rtl/ | Verilog AxMAC; synthesized & verified on EP4CE10 FPGA at 50 MHz |

6. Results Achieved

343 pytest tests pass; all six Python modules fully covered.

Bias accumulation confirmed: trunc error ∝N; stochastic ∝√N (N = 1… 4096).

Contribution A — INT8 K = 6 bias: trunc +29.9 → round −1.7 → stochastic −0.8.

Contribution B — non-uniform K cuts logit NRMSE up to 48 % at matched energy.

Cross-format Pareto: 8 precisions, global + per-format fronts (10 CSV files).

RTL hardware: AxMAC synthesized at 50 MHz on EP4CE10; board LED demo verified.

7. Deliverables

Eight Python modules with 343 unit tests; full (precision, K, rounding) CSV result tables; global and per-format Pareto figures covering 8 precision formats; Verilog RTL verified on EP4CE10 FPGA; final report covering baseline reproduction, deficiency analysis, Contribution A/B evaluation, and FP8 modernization.